As Built Roads (ABR)

ESF Submission Guide

| Client: | Ministry of Forests Information Management Group |
| --- | --- |
| Date: | May 21, 2004 |
| Revision: | 0 |

For Use with version 0 (Draft) of the ABR Schema

Vivid Solutions Inc.

Suite #1A, 2328 Government St.

Victoria, BC V8T 5G5

Phone:  (250) 385-6040

Fax:  (250) 385-6046

Website:  www.vividsolutions.com

Version Control

| REVISION NUMBER | DATE OF ISSUE | AUTHOR(S) | DESCRIPTION |
| --- | --- | --- | --- |
| 0 | May 10, 2004 | David Simpson | Original draft -- May 20th -- Changed Submission Item and Spatial Metadata sections to conform to schema changes |

Table of Contents

1. Introduction	5

1.1	This Document	5

1.2	What is a Submission?	5

1.3	XML and GML?	6

1.4	Who can Make a Submission?	6

1.5	How Do I Make a Submission?	7

1.6	Glossary	7

2. The Schema	9

2.1	Overview	9

2.2	Conventions	10

2.3	Data Types	10

2.3.1	anyURI	10

2.3.2	Boolean	10

2.3.3	Code Types	11

2.3.4	Date	11

2.3.5	gYear	11

2.3.6	Telephone Numbers	11

2.3.7	Geometry	11

2.4	Tips	13

3. Submission Components	14

3.1	Submission Header	14

3.2	Submission Metadata	14

3.3	Submission Content	15

3.3.1	ABR Metadata	15

3.3.2	Submission Item (Road Tenure)	15

4. Validation Rules	23

4.1	External system validations	23

4.2	Placement of Events	23

4.3	Class/Type relationship	23

5. Appendix A - Code Tables	24

6. Appendix B – An Example Submission	26

7. Appendix C – Troubleshooting	30

# Introduction

## This Document

This document is a guide for assembling As Built Roads data for submission via the Ministry of Forest’s Electronic Submission Framework (ESF).  It is intended to assist submission authors in producing valid submission documents and as a guide for developers that will be creating automated tools for this purpose.

Each component of a submission is described in its own section with each field therein explained in terms of its type and purpose. Fields requiring predefined values are identified and tables of acceptable values are supplied in the Appendices. The components of a valid As Built Roads (ABR) submission are described in the order they should appear in a submission document and examples are provided for each.

The submission schema, sample files and this guide are available for download on the ESF informational website: http://www.for.gov.bc.ca/his/esf/

## What is a Submission?

A submission is an XML/GML document that is uploaded via the ESF website for processing by application specific systems, in this case ABR. A submission can contain both attribute and spatial data in a single file and is structured according to a pre-defined schema.

As shown in the following diagram, a submission is first uploaded to the ESF website (the Submission Broker). Simple validation occurs at this point – i.e. ensuring the content adheres to the schema definition. The submission is then passed to the Business Agent, which performs business-specific validation of the content according to the rules described later in this document

Figure 1 - Submission Overview

The Business Agent interacts with an ESF Agent, which tracks information about the transaction – e.g. who submitted; when; whether it passed or failed; etc – and with any “downstream” systems or repositories. In this case, successfully validated data is loaded into the Operational Spatial Database (OSDB). If errors occur the ESF Agent sends appropriate messages back to the Submission Website where client can monitor the progress of their submissions.

It is important to note that ESF does not include the ability to create submissions. It only provides the mechanisms for submitting, validating and managing them. Creation of the documents is the responsibility of the users.

## XML and GML?

EXtensible Markup Language (XML) is a language for representing structured data. It uses a format that is easily readable by both humans and computers and is a standard supported by the W3C, the standards body for the World Wide Web. It has emerged as the primary format for encoding structured information and many tools are available for authoring, translating and viewing XML documents.

Geography Markup Language (GML) is a subset of XML that allows for the encoding of geographic information, including both the spatial and non-spatial properties of geographic features.

There are many XML resources available on the web and several are recommended on the ESF informational website.

## Who can Make a Submission?

There are several unique types of users that will be submitting As Built Roads data. Each will have restrictions on what they can submit, as listed in the following table:

| User | Authentication |
| --- | --- |
| Licencees | Must own the road being submitted – i.e. the submitter is the same as is registered for the permit or tenure associated with the road |
| Forest Districts | Can submit any valid road, regardless of district boundaries |
| Forest Districts on behalf of Licencees | Can submit any valid road, regardless of district boundaries, as long as the road is owned by the represented licencee |
| Service Providers on behalf of Licencees or Forest Districts | Service providers must be authorized to represent the client and the road must adhere to the appropriate restrictions as listed above |

BCTS will be treated as a Forest District for the purposes of submission – i.e. they will be able to submit any road, regardless of administrative boundaries.

## How Do I Make a Submission?

- Create the document, following the guidelines and samples in this document (samples are also available for download on the ESF website)

- Logon to the ESF submission website and upload the document (note that you will need to have a valid IDIR or BCeID account to do this)

- The document structure and geometry data will be validated immediately upon upload. Messages are returned to the website indicating the status of the submission

- The submission is sent to the business agent that will apply further checks to the submission content. Again, messages are relayed back to the website where users can monitor its progress

- Post-validation processing occurs – in the case of ABR this includes loading the data into the Operational Spatial Database

Figure 2 - Submission Process

## Glossary

| Term | Definition |
| --- | --- |
| ESF | Electronic Submission Framework. A Ministry maintained website and infrastructure that supports the submission, validation and processing of information. |
| Event | Events are physical features that occur on or about the road such as bridges, culverts, etc. They are represented as points or linear features and associated with the road section. |
| FRMA | Forest Road Management Application. MoF’s road inventory management system. |
| FTA | Forest Tenures Application. A system for managing and maintaining Forestry Tenure data. |
| GML | The Geography Markup Language (GML) is a standard supported by the Open GIS Consortium. It is an XML encoding that allows for the transport and storage of geographic information; including both the spatial and non-spatial properties of geographic features. |
| MoF | Ministry of Forests. |
| MSRM | Ministry of Sustainable Resource Management. |
| On Block Road | On Block Roads are roads that are built under the authority of the harvesting tenure, e.g. a forest licence. They are not associated with a road permit but are deemed to be permanent access structures as they may not be able to be rehabilitated. |
| PoC | Point Of Commencement. The starting point of a road |
| PoT | Point Of Termination. The ending point of a road |
| Road Section | A road section is a contiguous piece oif a road having a Point of Commencement (PoC) and Point of Termination (PoT). It is comprised of a collection of contiguous line segments that connect the two. |
| Tenured Road | A tenured road is one that has been planned and has a valid reference in FTA (i.e. an exhibit A has been submitted). |
| XML | Extensible Markup Language (XML) is a markup language for documents which contain structured data. It is a standard supported by the W3C, the standards body for the World Wide Web. It is a format that is easily readable by both humans and computers.. |
| XML Schema | The XML Schema Definition Language (XSDL) is a way of describing and constraining the content of XML documents. It allows, in a clear and unambiguous way, descriptions of what kinds of XML documents can be used in a given application. |

# The Schema

## Overview

A schema describes the structure of a submission document – in other words, a set of rules for the submission. The schema identifies:

- What data can be contained in the document

- The order in which it must be presented

- Whether specific data elements are mandatory or optional

- Restrictions on specific content, for example enumerated lists

An overview of how an ABR submission is organized is provided in the following diagram. Each represented component is described in detail later in this document.

Figure 3 - Schema Organization

## Conventions

- Submission components are described in the Submission Content section along with examples and detailed descriptions of each field that is contained within them. Data types and indications of whether the component is required or optional are supplied. Note that omitting a required section or mandatory field will result in an invalid submission.

- In the examples, +/- signs indicate collapsed sub-components. These components are described individually, following the section in which they appear. Geometry subcomponents are described in the Data Types section.

- XML uses tags to delineate information. Tags begin with the field or section name in a set of angled brackets and end with the same text in another set, preceded by a slash. The content is contained between the two. For example:

<tag>

content

</tag>

Tags are often nested within each other to represent hierarchical information, for example:

<Car>

<manufacturer>Honda</manufacturer>

<options>

<colour>Red</colour>

<interior>Leather</interior>

</options>

</Car>

## Data Types

Data types are listed with maximum sizes in following parentheses where applicable, for example: Alphanumeric (3). Precision is identified by a further set of parentheses, for example: Decimal (6) (2).

There are several data types that have specific requirements. The following sections describe the proper syntax and formatting to be used when entering content for these types.

### anyURI

This is a standard XML data type for representing a Uniform Resource Identifier. For the purposes of ABR they are used for email addresses; the “mailto:” prefix should be used as in the example:

<rst:emailAddress>mailto:John.Doe@MajorLicensee.com</rst:emailAddress>

### Boolean

Boolean (logical) values can be given as either 1/0 or true/false.   The name of a Boolean field always indicates the effect of a true value.  Thus  “outstandingObligation” indicates that if the field has a value of true, there is an obligation outstanding.

<rst:validationIndicator>1</rst:validationIndicator>

### Code Types

Data types that have the suffix “Type” or “Code” can contain only predefined values.  Valid content for these types is defined in the Code Tables appendix.

### Date

Dates must always be given in the format YYYY-MM-DD.  Thus 2003-04-10 indicates April 10, 2003.

<rst:declarationDate>2002-05-10</rst:declarationDate>

### gYear

This is a standard XML data type for representing a specific Gregorian calendar year. Years are represented as a positive, four-digit number.

<rst:fislcaYear>2002</rst:fiscalYear>

### Telephone Numbers

Telephone numbers use the custom data type, “TelephoneNumberType”. A valid telephone number must be 10 digits, starting with the area code and contain no separators.

<rst:phonenumber>6042515612</rst:phonenumber>

### Geometry

Geometry is required for each item (road section) in a submission and for each event associated with the section. It includes two components: spatial metadata (information about the data, for example how it was captured) and the feature itself (represented as GML).

#### Spatial Metadata

Spatial metadata is required for each road section in a submission.

<mof:spatialMetaData>

<mof:dataSource>datasource</mof:dataSource>

<mof:mostRecentSourceDate>2003-01-01</mof:mostRecentSourceDate>

<mof:captureMethod>capture method</mof:captureMethod>

<mof:observationDate>2003-01-01</mof:observationDate>

<mof:dataAccuracy>accuracy</mof:dataAccuracy>

<mof:description>description</mof:description>

<mof:ccsmCode>GA9100032</mof:ccsmCode>

<mof:horizontalAccuracy>15</mof:horizontalAccuracy>

<mof:verticalAccuracy>10</mof:verticalAccuracy>

<mof:horizontalDatum>nad83</mof:horizontalDatum>

<mof:verticalDatum>cvd28</mof:verticalDatum>

</mof:spatialMetaData>

#### GML

Road sections are represented as line strings (a collection of points representing the centreline) and are written as comma-delimited pairs separated by spaces.

<abr:centreLineOf>

<gml:LineString srsName="EPSG:42102">

<gml:coordinates>3651.8,1109.5 4056.9,1238.9 3663.0,1514.6 4085.0,1734.0</gml:coordinates>

</gml:LineString>

</abr:centreLineOf>

Event locations are defined as either a single set of coordinates or as distances (in metres) from the road’s point of commencement (PoC). Either can be used for a given event:

<abr:location>

<abr:coordinate>

<gml:Point srsName="EPSG:42012">

<gml:coord>

<gml:X>30</gml:X>

<gml:Y>40</gml:Y>

</gml:coord>

</gml:Point>

</abr:coordinate>

</abr:location>

<abr:location>

<abr:distance>

<abr:from>10.3</abr:from>

<abr:to>10.4</abr:to>

</abr:distance>

</abr:location>

Both the from and to distances are given in metres from the PoC, e.g. the example above states that the event is located 10.3 metres from the beginning of the road section and extends to 10.4 metres from it. The data type is Decimal (18)(2).

Note that, if given in coordinates, events will be validated in relation to their distance from the centreline. Events must be located within 10 meters of the road and submissions outside that tolerance will fail. Also note that all location data will be converted to distances from the PoC for storage in the database.

A Note on Spatial Coordinate Systems:

Each road section must specify the coordinate system used to represent it.  The “srsName” attribute within the gml:LineString tag is used to define this.  Note that it is permissible (but not recommended) that different coordinate systems can be used within the same submission.  Refer to the following table in the Code Tables for valid entries.

| SRS Name | Coordinate System |
| --- | --- |
| EPSG:4326 | WGS 84 (Geographics) |
| EPSG:32607 | WGS 84 / UTM zone 7N |
| EPSG:32608 | WGS 84 / UTM zone 8N |
| EPSG:32609 | WGS 84 / UTM zone 9N |
| EPSG:32610 | WGS 84 / UTM zone 10N |
| EPSG:32611 | WGS 84 / UTM zone 11N |
| EPSG:42102 | NAD83 BC Albers |

## Tips

- Missing or misnamed tags are a common problem. Check that all your tags have corresponding end tags before submitting

- XML is case sensitive. If a tag is expected to read “smallCase”, but is entered as “smallcase”, the validation will fail

- For more information on XML and GML, refer to the ESF website where you can find links to several useful resources (http://www.for.gov.bc.ca/his/esf/).

# Submission Components

The following sections describe each logical component of a submission document. Refer to Appendix B for an example of a complete submission.

## Submission Header

This section is required. Each submission document starts with information necessary to carry out correct XML processing.  This section will be identical for each submission document, with the possible exception of the version number.

The Submission Header defines several things:

- The element esf:ESFSubmission tag indicates that this is a submission to be processed via the ESF

- The namespace (xmlns) tags define the paths to various reference information used during processing

- The schemaLocation attribute associates the namespaces with a URL that indicates the version of the schema being used

Note that he version specified in the header must match the version of the schema you are using, i.e. the schema you downloaded from the website.

<?xml version="1.0" encoding="UTF-8" ?>

<esf:ESFSubmission xmlns:esf="http://www.for.gov.bc.ca/schema/esf" xmlns:gml="http://www.opengis.net/gml" xmlns:xlink="http://www.w3.org/1999/xlink" xmlns:mof="http://www.for.gov.bc.ca/schema/base" xmlns:abr="http://www.for.gov.bc.ca/schema/abr" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="http://www.for.gov.bc.ca/schema/esf xsd/MOF/esf-submission.xsd http://www.for.gov.bc.ca/schema/abr xsd/MOF/mof-abr.xsd">

+ <esf:submissionMetadata>

<esf:emailAddress>mailto:John.Doe@MajorLicensee.com</esf:emailAddress>

<esf:telephoneNumber>6044445555</esf:telephoneNumber>

</esf:submissionMetadata>

+ <esf:submissionContent>

- <abr:ABRSubmission>

<gml:description>A sample submission of As-Built Roads</gml:description>

<gml:name>Optional name.</gml:name>

- <gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

- <abr:submissionMetadataProperty>

- <abr:SubmissionMetadata>

<abr:name>name</abr:name>

<abr:submissionDate>2003-01-01</abr:submissionDate>

<abr:action>I</abr:action>

<abr:clientCode>12345678</abr:clientCode>

<abr:clientLocationCode>00</abr:clientLocationCode>

</abr:SubmissionMetadata>

</abr:submissionMetadataProperty>

- <abr:submissionItem>

- <abr:RoadTenure>

- <gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

<abr:forestFileID>R12345</abr:forestFileID>

- <abr:roadSectionMember>

- <abr:RoadSection>

- <gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

- <mof:spatialMetaData>

<mof:dataSource>datasource</mof:dataSource>

<mof:mostRecentSourceDate>2003-01-01</mof:mostRecentSourceDate>

<mof:captureMethod>capture method</mof:captureMethod>

<mof:observationDate>2003-01-01</mof:observationDate>

<mof:dataAccuracy>accuracy</mof:dataAccuracy>

<mof:description>description</mof:description>

<mof:ccsmCode>GA9100032</mof:ccsmCode>

<mof:horizontalAccuracy>15</mof:horizontalAccuracy>

<mof:verticalAccuracy>10</mof:verticalAccuracy>

<mof:horizontalDatum>nad83</mof:horizontalDatum>

<mof:verticalDatum>cvd28</mof:verticalDatum>

</mof:spatialMetaData>

<abr:roadSectionID>1</abr:roadSectionID>

<abr:cuttingPermitID />

- <abr:centreLineOf>

- <gml:LineString srsName="EPSG:42102">

<gml:coordinates>0,0 100,0</gml:coordinates>

</gml:LineString>

</abr:centreLineOf>

- <abr:eventItem>

- <abr:bridge>

<abr:signageDate>2003-01-01</abr:signageDate>

<abr:revegDate>2003-01-01</abr:revegDate>

<abr:comments>comments</abr:comments>

</abr:deactivation>

</abr:eventItem>

</abr:RoadSection>

</abr:roadSectionMember>

</abr:RoadTenure>

</abr:submissionItem>

</abr:ABRSubmission>

</esf:submissionContent>

</esf:ESFSubmission>

## Submission Metadata

This is a required section. The submission metadata includes contact information for the submitting party. This is required to process the submission.

<esf:submissionMetadata>

<esf:emailAddress>mailto:John.Doe@MajorLicensee.com</esf:emailAddress>

<esf:telephoneNumber>6044445555</esf:telephoneNumber>

</esf:submissionMetadata>

| Field | TYPE | Description |
| --- | --- | --- |
| esf:emailAddress | anyURI | Required. The email address of the person or organization submitting. Refer to the Data Types section for further description of this type. |
| esf:telephoneNumber | TelephoneNumberType | Required. The telephone number of the person or organization submitting. Refer to the Data Types section for further description of this type. |

## Submission Content

Required section. This is the container for the submission details. It includes required structural tags as well as some optional descriptive information.

<esf:submissionContent>

<abr:ABRSubmission>

<gml:description>A sample submission of As-Built Roads</gml:description>

<gml:name>Optional name.</gml:name>

<gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

+ <abr:submissionMetadataProperty>

- <abr:SubmissionMetadata>

<abr:name>name</abr:name>

<abr:submissionDate>2003-01-01</abr:submissionDate>

<abr:action>I</abr:action>

<abr:clientCode>12345678</abr:clientCode>

<abr:clientLocationCode>00</abr:clientLocationCode>

</abr:SubmissionMetadata>

</abr:submissionMetadataProperty>

+ <abr:submissionItem>

- <abr:RoadTenure>

- <gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

<abr:forestFileID>R12345</abr:forestFileID>

- <abr:roadSectionMember>

- <abr:RoadSection>

- <gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

- <mof:spatialMetaData>

<mof:dataSource>datasource</mof:dataSource>

<mof:mostRecentSourceDate>2003-01-01</mof:mostRecentSourceDate>

<mof:captureMethod>capture method</mof:captureMethod>

<mof:observationDate>2003-01-01</mof:observationDate>

<mof:dataAccuracy>accuracy</mof:dataAccuracy>

<mof:description>description</mof:description>

<mof:ccsmCode>GA9100032</mof:ccsmCode>

<mof:horizontalAccuracy>15</mof:horizontalAccuracy>

<mof:verticalAccuracy>10</mof:verticalAccuracy>

<mof:horizontalDatum>nad83</mof:horizontalDatum>

<mof:verticalDatum>cvd28</mof:verticalDatum>

</mof:spatialMetaData>

<abr:roadSectionID>1</abr:roadSectionID>

<abr:cuttingPermitID />

- <abr:centreLineOf>

- <gml:LineString srsName="EPSG:42102">

<gml:coordinates>0,0 100,0</gml:coordinates>

</gml:LineString>

</abr:centreLineOf>

- <abr:eventItem>

- <abr:bridge>

<abr:signageDate>2003-01-01</abr:signageDate>

<abr:revegDate>2003-01-01</abr:revegDate>

<abr:comments>comments</abr:comments>

</abr:deactivation>

</abr:eventItem>

</abr:RoadSection>

</abr:roadSectionMember>

</abr:RoadTenure>

</abr:submissionItem>

</abr:ABRSubmission>

</esf:submissionContent>

| Field | TYPE | Description |
| --- | --- | --- |
| abr:ABRSumission | N/A | Required tag. |
| gml:description | String | Optional. High level description of the submission |
| gml:name | String | Optional. A descriptive name for the submission |
| gml:boundedBy | BoundingShapeType | Required. GML requires a bounding box to be specified. This bounding box is not used by ESF and should be set to null, as in the example. |

### ABR Metadata

Required section. The ABR metadata is used to provide information about the submission itself. In the case of ABR it is used to indicate the way in which the submission should be processed.

<abr:submissionMetadataProperty>

<abr:SubmissionMetadata>

<abr:validateOnly>true</abr:validateOnly>

<abr:contactName>Licensee contact Name</abr:contactName>

</abr:SubmissionMetadata>

</abr:submissionMetadataProperty>

| Field | TYPE | Description |
| --- | --- | --- |
| abr:SubmissionMetadata | N/A | Required tag. |
| abr:validateOnly | Boolean | Required. Indicates whether the submission should be validated without loading it to the database. False indicates that the submission will be loaded upon successful validation. |
| abr:contactName | AlphaNumeric (50) | Required. The name of the licensee contact. |

### Submission Item (Road Tenure)

Required section. Submissions must contain at least one submission item. Each submission item must identify a Road Tenure or Cut Block and (optionally) Cutting Permit with which it is associated.

<abr:submissionItem>

<abr:RoadTenure>

<gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

<abr:forestFileId>A12345</abr:forestFileId>

<abr:timberMark>F16110</abr:timberMark>

<abr:cutBlockId>A</abr:cutBlockId>

+ <abr:roadSectionMember>

- <abr:RoadSection>

- <gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

- <mof:spatialMetaData>

<mof:dataSource>datasource</mof:dataSource>

<mof:mostRecentSourceDate>2003-01-01</mof:mostRecentSourceDate>

<mof:captureMethod>capture method</mof:captureMethod>

<mof:observationDate>2003-01-01</mof:observationDate>

<mof:dataAccuracy>accuracy</mof:dataAccuracy>

<mof:description>description</mof:description>

<mof:ccsmCode>GA9100032</mof:ccsmCode>

<mof:horizontalAccuracy>15</mof:horizontalAccuracy>

<mof:verticalAccuracy>10</mof:verticalAccuracy>

<mof:horizontalDatum>nad83</mof:horizontalDatum>

<mof:verticalDatum>cvd28</mof:verticalDatum>

</mof:spatialMetaData>

<abr:roadSectionID>1</abr:roadSectionID>

- <abr:centreLineOf>

- <gml:LineString srsName="EPSG:42102">

<gml:coordinates>0,0 100,0</gml:coordinates>

</gml:LineString>

</abr:centreLineOf>

- <abr:eventItem>

- <abr:bridge>

<abr:signageDate>2003-01-01</abr:signageDate>

<abr:revegDate>2003-01-01</abr:revegDate>

<abr:comments>comments</abr:comments>

</abr:deactivation>

</abr:eventItem>

</abr:RoadSection>

</abr:roadSectionMember>

</abr:RoadTenure>

</abr:submissionItem>

| Field | TYPE | Description |
| --- | --- | --- |
| abr:RoadTenure | N/A | Required tag. |
| gml:boundedBy | BoundingShapeType | Required. GML requires a bounding box to be specified. This bounding box is not used by ESF and should be set to null, as in the example. |
| abr:forestFileId | AlphaNumeric (10) | Required. The Forest File Id of the tenure. |
| abr:timberMark | AlphaNumeric (6) | Optional. The Timber Mark Id. |
| abr:cutBlockId | AlphaNumeric (10) | Required. The cut block Id. |

#### Road Section

Required section. Each submission must contain at least one road section. Sections must be uniquely identified with respect to the Road Tenure – i.e. if two or more sections are supplied with the same roadSectionId the validation will fail.

<abr:roadSectionMember>

<abr:RoadSection>

<gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

+ <mof:spatialMetaData>

<mof:dataSource>datasource</mof:dataSource>

<mof:mostRecentSourceDate>2003-01-01</mof:mostRecentSourceDate>

<mof:captureMethod>capture method</mof:captureMethod>

<mof:observationDate>2003-01-01</mof:observationDate>

<mof:dataAccuracy>accuracy</mof:dataAccuracy>

<mof:description>description</mof:description>

<mof:ccsmCode>GA9100032</mof:ccsmCode>

<mof:horizontalAccuracy>15</mof:horizontalAccuracy>

<mof:verticalAccuracy>10</mof:verticalAccuracy>

<mof:horizontalDatum>nad83</mof:horizontalDatum>

<mof:verticalDatum>cvd28</mof:verticalDatum>

</mof:spatialMetaData>

<abr:roadSectionId>1</abr:roadSectionId>

+ <abr:centreLineOf>

- <gml:LineString srsName="EPSG:42102">

<gml:coordinates>0,0 100,0</gml:coordinates>

</gml:LineString>

</abr:centreLineOf>

+ <abr:eventItem>

- <abr:bridge>

<abr:plannedActivity>true</abr:plannedActivity>

<abr:seasonCode>abc</abr:seasonCode>

<abr:oustandingObligation>true</abr:oustandingObligation>

<abr:inspectionPlanned>true</abr:inspectionPlanned>

<abr:comments>comments</abr:comments>

</abr:constructionDate>

</abr:eventItem>

+ <abr:eventItem>

- <abr:accessControl>

<abr:closureId>1234567890123456</abr:closureId>

<abr:installDate>2003-01-01</abr:installDate>

<abr:objectiveCode>obj code</abr:objectiveCode>

<abr:vehicleUse>veh use</abr:vehicleUse>

</abr:accessControl>

</abr:eventItem>

+ <abr:eventItem>

- <abr:deactivation>

<abr:signageDate>2003-01-01</abr:signageDate>

<abr:revegDate>2003-01-01</abr:revegDate>

<abr:comments>comments</abr:comments>

</abr:deactivation>

</abr:eventItem>

</abr:RoadSection>

</abr:roadSectionMember>

| Field | TYPE | Description |
| --- | --- | --- |
| abr:RoadSection | N/A | Required tag. |
| gml:boundedBy | BoundingShapeType | Required. GML requires a bounding box to be specified. This bounding box is not used by ESF and should be set to null, as in the example. |
| abr:roadSectionId | Alphanumeric (10) | Required. A user-supplied identifier for the road section. Must be unique within the Road Tenure. |
| abr:centreLineOf | N/A | Required. See Data Types – Geometry for description and example. |

##### Spatial Metadata

Required section. Each road section will have associated metadata.

<mof:spatialMetaData>

<mof:dataSource>datasource</mof:dataSource>

<mof:mostRecentSourceDate>2003-01-01</mof:mostRecentSourceDate>

<mof:captureMethod>capture method</mof:captureMethod>

<mof:observationDate>2003-01-01</mof:observationDate>

<mof:dataAccuracy>75</mof:dataAccuracy>

<mof:description>description</mof:description>

<mof:ccsmCode>GA9100032</mof:ccsmCode>

<mof:horizontalAccuracy>15</mof:horizontalAccuracy>

<mof:verticalAccuracy>10</mof:verticalAccuracy>

<mof:horizontalDatum>nad83</mof:horizontalDatum>

<mof:verticalDatum>cvd28</mof:verticalDatum>

<mof:coordinateSystemCode>1</mof:coordinateSystemCode>

</mof:spatialMetaData>

| Field | TYPE | Description |
| --- | --- | --- |
| mof:dataSource | Alphanumeric (10) | Optional. The source of the spatial data, e.g. Field Survey, Air Photos, etc. |
| mof:mostRecentSourceDate |  | Optional. Most recent collection date of the source data |
| mof:captureMethod | Alphanumeric (30) | Optional. The method used to capture the spatial data, e.g. Differential GPS, Tablet Digitizing, etc. |
| mof:observationDate | Date | Optional. The data the observation was made. This applies to the data. For example the data of aerial photography or the data of the survey. |
| mof:dataAccuracy | Alphanumeric (255) | Optional. |
| mof:description | Alphanumeric (2000) | Optional. A description that can be used to clarify any of the values in the spatial meta-data. |
| mof:ccsmCode | Alphanumeric (10) | Optional. The TRIM CCSM code which best describes the feature. Ex: GA9100032 |
| mof:horizontalAccuracy | Integer (3) | Optional. The horizontal accuracy bound. |
| mof:verticalAccuracy | Integer (3) | Optional. The vertical accuracy bound. |
| mof:horizontalDatum | Alphanumeric (10) | Optional. The horizontal datum. Must be one of:Nad83, nad27. |
| mof:verticalDatum | Alphanumeric (10) | Optional. The vertical datum. In British Columbia most vertical data uses the Canadian Vertical Datum. Ex: cvd28 |

##### Centre Line

Required section. Road section geometry consists of a line string indicating the centreline of the road.

<abr:centreLineOf>

+ <gml:LineString srsName="EPSG:42102">

</abr:centreLineOf>

| Field | TYPE | Description |
| --- | --- | --- |
| gml:LineString | N/A | Required tag. Refer to the Data Types - Geometry section for description, example and acceptable srsName values. |

##### Event Item (Bridge)

Optional section. Bridges are defined as structures on a road that pass over water bodies or other obstacles.

<abr:eventItem>

<abr:bridge>

+ <abr:location>

<abr:superStructureIdNo>12345678</abr:superStructureIdNo>

<abr:siteNumber>12345678</abr:siteNumber>

<abr:designLoadRating>100</abr:designLoadRating>

<abr:superStructureType>type</abr:superStructureType>

<abr:substructureType>abc</abr:substructureType>

<abr:deckType>abc</abr:deckType>

<!-- Optional elements  -->

<abr:fishPresent>true</abr:fishPresent>

<abr:riparianClass>true</abr:riparianClass>

<abr:fieldKmMarker>5</abr:fieldKmMarker>

<abr:streamName>stream name</abr:streamName>

<abr:currentLoadRating>100</abr:currentLoadRating>

<abr:designVehicleCode>abc</abr:designVehicleCode>

<abr:nextInspectionDate>2003-01-01</abr:nextInspectionDate>

<abr:bridgeDeckWidth>1</abr:bridgeDeckWidth>

<abr:chronicMaintenance>true</abr:chronicMaintenance>

<abr:upstreamRisk>true</abr:upstreamRisk>

<abr:inletRisk>true</abr:inletRisk>

<abr:adequateSize>true</abr:adequateSize>

<abr:historicWashout>true</abr:historicWashout>

</abr:bridge>

</abr:eventItem>

| Field | TYPE | Description |  |
| --- | --- | --- | --- |
| abr:bridge | N/A | Required tag. |  |
| abr:location | N/A | Required. See Data Types – Geometry for description and example. |  |
| abr:superStructureIdNo | Alphanumeric (8) | Required. A unique bridge ID number. |  |
| abr:siteNumber | Alphanumeric (8) | Required. A unique ID for the crossing location. |  |
| abr:designLoadRating | Integer (10) | Required. The deigned load rating in tonnes. |  |
| abr:superStructureType | Alphanumeric (10) | Required. Bridge superstructure type. Refer to the Code Tables appendix for a list of values. |  |
| abr:substructureType | Alphanumeric (14) | Required. Bridge substructure type. Refer to the Code Tables appendix for a list of values. |  |
| abr:deckType | Alphanumeric (20) | Required. Bridge deck type. Refer to the Code Tables appendix for a list of values. |  |
| abr:fishPresent | Boolean | Optional. True indicates that the underlying water body is fish bearing. |  |
| abr:riparianClass | Alphanumeric (10) | Optional. Riparian class of the underlying stream. . Refer to the Code Tables appendix for a list of values. |  |
| abr:fieldKmMarker | Alphanumeric (15) | Optional. Marker indicating the location along the route |  |
| abr:streamName | Alphanumeric (30) | Optional. The name of the underlying body of water. |  |
| abr:currentLoadRating | Integer (10) | Optional. Current load rating of the bridge in tonnes. |  |
| abr:designVehicleCode | Alphanumeric (10) | Optional. Design vehicle code. Refer to the Code Tables appendix for a list of values. |  |
| abr:nextInspectionDate | Date | Optional. Date of the next scheduled inspection. |  |
| abr:bridgeDeckWidth | Decimal (7)(2) | Optional. Width of the deck in metres. |  |
| abr:chronicMaintenance | Boolean | Optional. True indicates that the bridge requires chronic maintenance. |  |
| abr:upstreamRisk | Boolean | Optional. True indicates there is a risk to the structure from upstream. |  |
| abr:inletRisk | Boolean | Optional. True indicates there is a risk to the inlet area of the structure. |  |
| abr:adequateSize | Boolean | Optional. True indicates the structure is of adequate size. |  |
| abr:historicWashout | Boolean | Optional. True indicates the structure has washed out in the past. |  |

##### Event Item (Culvert)

Optional section. Culverts are defined as pipes, arches or boxes, or log structures, not greater than 6 m in span that are located below the road surface of a road and are designed to carry water from one side of the road to the other.

<abr:eventItem>

<abr:culvert>

+ <abr:location>

<abr:siteStructureNumber>1234567890</abr:siteStructureNumber>

<abr:diameterOrSpan>5000</abr:diameterOrSpan>

<abr:materialCode>abc</abr:materialCode>

<abr:designLoadRating>10</abr:designLoadRating>

<!-- Optional elements  -->

<abr:fishPresent>true</abr:fishPresent>

<abr:riparianClass>abc</abr:riparianClass>

<abr:height>5000</abr:height>

<abr:streamName>stream name</abr:streamName>

<abr:currentLoadRating>10</abr:currentLoadRating>

<abr:designVehicleCode>abc</abr:designVehicleCode>

<abr:length>10</abr:length>

<abr:description>description</abr:description>

<abr:adequateSize>true</abr:adequateSize>

<abr:inletRisk>true</abr:inletRisk>

<abr:chronicMaintenance>true</abr:chronicMaintenance>

<abr:comments>comments</abr:comments>

<abr:siteNumber>1234567890</abr:siteNumber>

<abr:historicWashout>true</abr:historicWashout>

</abr:culvert>

</abr:eventItem>

| Field | TYPE | Description |  |
| --- | --- | --- | --- |
| abr:culvert | N/A | Required tag. |  |
| abr:location | N/A | Required. See Data Types – Geometry for description and example. |  |
| abr:siteStructureNumber | Alphanumeric (10) | Required. Unique MoF structure number. |  |
| abr:diameterOrSpan | Decimal (9)(3) | Required. Diameter for round, width for others. |  |
| abr:materialCode | Alphanumeric (10) | Required. Type of material comprising the structure. Refer to the Code Tables appendix for a list of values. |  |
| abr:designLoadRating |  | Required. The deigned load rating in tonnes. |  |
| abr:fishPresent | Boolean | Optional. True indicates that the underlying water body is fish bearing. |  |
| abr:riparianClass |  | Optional. Riparian class of the underlying stream. . Refer to the Code Tables appendix for a list of values. |  |
| abr:height | Decimal (7)(2) | Optional. Height of the opening in mm. |  |
| abr:streamName |  | Optional. The name of the underlying body of water. |  |
| abr:currentLoadRating | Integer | Optional. Current load rating of the culvert in tonnes. |  |
| abr:designVehicleCode |  | Optional. Design vehicle code. Refer to the Code Tables appendix for a list of values. |  |
| abr:length | Decimal (7)(2) | Optional. Length along the stream in mm. |  |
| abr:description | Alphanumeric (80) | Optional. Description of the culvert. |  |
| abr:adequateSize | Boolean | Optional. True indicates that the structure is of adequate size. |  |
| abr:inletRisk | Boolean | Optional. True indicates there is a risk to the inlet area of the structure. |  |
| abr:chronicMaintenance | Boolean | Optional. True indicates that the bridge requires chronic maintenance. |  |
| abr:comments |  | Optional. Comments regarding the culvert. |  |
| abr:siteNumber | Alphanumeric (10) | Required. A unique ID for the crossing location. |  |
| abr:historicWashout | Boolean | Optional. True indicates the structure has washed out in the past. |  |

##### Event Item (Linear Structure)

Optional section. Retaining wall or french drain.

<abr:eventItem>

<abr:nonStreamCrossingEngineeredStructure>

+ <abr:location>

<abr:type>abc</abr:type>

<abr:engineeredStructure>true</abr:engineeredStructure>

<abr:Class>FD</abr:Class>

</abr:nonStreamCrossingEngineeredStructure>

</abr:eventItem>

| Field | TYPE | Description |
| --- | --- | --- |
| abr: | N/A | Required tag. |
| abr:location | N/A | Required. See Data Types – Geometry for description and example. |
| abr:type | Alphanumeric (20) | Required. The structure type code. Refer to the Code Tables appendix for a list of values. |
| abr:engineeredStructure | Boolean | Required. True indicates the structure was engineered. |
| abr:Class | Alphanumeric (20) | Required. The structure class. Refer to the Code Tables appendix for a list of values. |

##### Event Item (Organic Material)

Optional section. Organic material represents road sections containing stumps, roots, and embedded logs left or placed:

  - under the road fill within the road prism width, if the road is locationed on landslide prone terrain, or

  - under the travelled portion of the road fill for other road locations.

<abr:eventItem>

<abr:organicMaterial>

+ <abr:location>

<abr:featureType>abc</abr:featureType>

</abr:organicMaterial>

</abr:eventItem>

| Field | TYPE | Description |
| --- | --- | --- |
| abr: | N/A | Required tag. |
| abr:location | N/A | Required. See Data Types – Geometry for description and example. |
| abr:featureType | Alphanumeric (20) | Required. The type of feature. Refer to the Code Tables appendix for a list of values. |

##### Event Item (Construction Date)

Optional section. Construction date includes information about when the event occurred or when construction of it was completed.

<abr:eventItem>

<abr:constructionDate>

+ <abr:location>

<abr:activityCode>abc</abr:activityCode>

<abr:actualCompletionDate>2003-01-01</abr:actualCompletionDate>

<!-- Optional elements  -->

<abr:plannedActivity>true</abr:plannedActivity>

<abr:seasonCode>abc</abr:seasonCode>

<abr:oustandingObligation>true</abr:oustandingObligation>

<abr:inspectionPlanned>true</abr:inspectionPlanned>

<abr:comments>comments</abr:comments>

</abr:constructionDate>

</abr:eventItem>

| Field | TYPE | Description |
| --- | --- | --- |
| abr: | N/A | Required tag. |
| abr:location | N/A | Required. See Data Types – Geometry for description and example. |
| abr:activityCode | Alphanumeric (10) | Required. The type of construction activity. Refer to the Code Tables appendix for a list of values. |
| abr:actualCompletionDate | Date | Required. Date the construction was completed. |
| abr:plannedActivity | Boolean | Optional. True indicates it was a planned activity. |
| abr:seasonCode | Alphanumeric (10) | Optional. Seasonal constraints. Refer to the Code Tables appendix for a list of values. |
| abr:oustandingObligation | Boolean | Optional. True indicates there are outstanding obligations in relation to the activity. |
| abr:inspectionPlanned | Boolean | Optional. True indicates an inspection is planned. |
| abr:comments |  | Optional. Comments regarding the construction activity. |

##### Event Item (Access Control)

Optional section. Access control refers to features located on the road to prevent or restrict access, such as gates and berms.

<abr:eventItem>

<abr:accessControl>

+ <abr:location>

<abr:accessControlFeature>abc</abr:accessControlFeature>

<!-- Optional elements  -->

<abr:closureId>1234567890123456</abr:closureId>

<abr:installDate>2003-01-01</abr:installDate>

<abr:objectiveCode>obj code</abr:objectiveCode>

<abr:vehicleUse>veh use</abr:vehicleUse>

</abr:accessControl>

</abr:eventItem>

| Field | TYPE | Description |
| --- | --- | --- |
| abr: |  | Required tag. |
| abr:location | N/A | Required. See Data Types – Geometry for description and example. |
| abr:accessControlFeature | Alphanumeric (20) | Required. Type of access feature. Refer to the Code Tables appendix for a list of values. |
| abr:closureId | Alphanumeric (20) | Optional. An assigned structure ID. |
| abr:installDate | Date | Optional. Date the structure was installed. |
| abr:objectiveCode | Alphanumeric (10) | Optional. The intent of the access control feature. Refer to the Code Tables appendix for a list of values. |
| abr:vehicleUse | Alphanumeric (10) | Optional. The intended vehicle access. Refer to the Code Tables appendix for a list of values. |

##### Event Item (Deactivation)

Optional section. Deactivation refers to a permanent access structure (road) that has been deactivated in accordance with the prescribed requirements: barricade the road surface, remove bridge and log superstructures and stream pipe culverts, remove bridge and log culvert substructures, if failure would have a material adverse effect, stabilize the road prism and clearing width of the road.

<abr:eventItem>

<abr:deactivation>

+ <abr:location>

<abr:deactivationlevel>abc</abr:deactivationlevel>

<abr:actualCompletionDate>2003-01-01</abr:actualCompletionDate>

<!-- Optional elements  -->

<abr:signageDate>2003-01-01</abr:signageDate>

<abr:revegDate>2003-01-01</abr:revegDate>

<abr:comments>comments</abr:comments>

</abr:deactivation>

</abr:eventItem>

| Field | TYPE | Description |
| --- | --- | --- |
| abr: |  | Required tag. |
| abr:location | N/A | Required. See Data Types – Geometry for description and example. |
| abr:deactivationlevel |  | Required. The degree of deactivation. Refer to the Code Tables appendix for a list of values. |
| abr:actualCompletionDate | Date | Required. Date the deactivation was complete. |
| abr:signageDate | Date | Optional. Date the signage was installed. |
| abr:revegDate | Date | Optional. Date that revegetation was done. |
| abr:comments |  | Optional. Comments regarding the deactivation. |

# Validation Rules

Submissions undergo validation in regard to their adherence to the schema as defined in this document. Exceptions such as missing mandatory data, misspelled tags names, etc. will cause validation errors. Additionally, there are some business specific validations that take place prior to a submission being accepted.

<This section to be completed>

## External system validations

Lookups for appropriate permits or related tenures

## Placement of Events

- Events must be located within 10M of the centreline of the road.

- An event distance is not greater than the length of the road

- An event's "to" distance is not less than the "from" distance

## Class/Type relationship

Engineered structures class/type relationship (see code tables)

# Appendix A - Code Tables

<This section to be completed>

Access Control

| Code | Code Description |
| --- | --- |
| BERM | Berm |
| BLDER | Boulders/Blocks |
| GATE | Locked gate |
| RNGATE | Range Gate |
| SIGN | Sign |
| STRCT | Bridge/Culvert removed |
| TRAP | Tank Trap |

Construction Activity

| Code | Code Description |
| --- | --- |
| NEW ROAD | New Road Construction |
| REACTIVATE | Road Reactivation |
| RELOCATE | Road Relocation |
| SURF | Surfacing |
| WIDEN | Road Widening |

Culvert Material

| Code | Code Description |
| --- | --- |
| CONC | Concrete |
| LOG | Log |
| MTL | CSP / CMP |
| OTH | Other |
| PLA | Plastic |

Deck Type

| Code | Code Description |
| --- | --- |
| C | Concrete |
| CC | Concrete Composite |
| G | Gravel |
| S | Steel |
| TT | Timber - Treated |
| TU | Timber - Untreated |

Design Vehicle

| Code | Code Description |
| --- | --- |
| L100 | Off Highway (L100) |
| L150 | Off Highway (L150) |
| L165 | Off Highway (L165) |
| L45 | Old On Highway (L45) |
| L60 | Old On Highway (L60) |
| L75 | On Highway (L75) |
| OTHER | Other |

Deactivation Level

| Code | Code Description |
| --- | --- |
| DP | Permanent |
| DS | Semi-Permanent |
| DT | Temporary |

Engineered Structures Class

| Code | Code Description |
| --- | --- |
| WALL | Wall |
| FD | French Drain |

Engineered Structures Type for Class = FD

| Code | Code Description |
| --- | --- |
| EP | Embedded pipe |
| RF | Rock Fill |

Engineered Structures Type for Class = WALL

| Code | Code Description |
| --- | --- |
| BLOCKS | Lock blocks |
| EARTH FILL | Earth fill |
| LOG CRIB | Log crib |
| OTHER | Other retaining wall |
| REIN CONCRETE | Reinforced concrete |
| ROCKFILL | Rockfill |
| STEEL BIN | Steel Binwall |
| STEEL PIPE | Steel pipe |

Objective

| Code | Code Description |
| --- | --- |
| DR | Damage to Road |
| EP | Environmental Protection |
| FH | Fire Hazard |
| LC | Livestock Control |
| US | User Safety |
| WP | Wildlife Protection |

Organic Material

| Code | Code Description |
| --- | --- |
| ORGANIC - PRISM |  |
| ORGANIC - TRAVL |  |

Riparian Class

| Code | Code Description |
| --- | --- |
| L1 | Lake: riparian class L1 |
| L2 | Lake: riparian class L2 |
| L3 | Lake: riparian class L3 |
| L4 | Lake: riparian class L4 |
| S1 | Stream: riparian class S1 |
| S2 | Stream: riparian class S2 x |
| S3 | Stream: riparian class S3 |
| S4 | Stream: riparian class S4 |
| S5 | Stream: riparian class S5 |
| S6 | Stream: riparian class S6 |
| UC | Unclassified waterbody requiring id & Mgt |
| W1 | Wetland: riparian class W1 |
| W2 | Wetland: riparian class W2 |
| W3 | Wetland: riparian class W3 |
| W4 | Wetland: riparian class W4 |
| W5 | Wetland: riparian class W5 |

Season

| Code | Code Description |
| --- | --- |
| AN | Any Season |
| FA | Fall |
| SP | Spring |
| SU | Summer |
| WI | Winter |
| DF | Dry or Frozen Soils |
| FW | Fish Windows |
| TS | Time Sensitive |

Superstructure Type

| Code | Code Description |  |
| --- | --- | --- |
| CONCRETE |  | Concrete |
| GLULAM |  | Glulam |
| STEEL |  | Steel |

Substructure Type

| Code | Code Description |  |
| --- | --- | --- |
| BIN |  | Binwall |
| CB |  | Concrete Block |
| CIP |  | Concrete Cast in Place |
| CP |  | Concrete Precast |
| CRIB |  | Crib log or Timber |
| O |  | Other |
| PP |  | Pile -Permanent Materials |
| PT |  | Pile - Temporary Materials |
| SP |  | Sill - Permanent Materials |
| SPP |  | Steel Pipe and Precast Pat |
| ST |  | Sill -Temporary Materials |

Vehicle Use

| Code | Code Description |  |
| --- | --- | --- |
| 2WD |  | 2 Wheel Drive |
| 4WD |  | 4 Wheel Drive |
| ATV |  | All Terrain Vechicles |
| FOOT |  | Foot/Bike/Hike |
| NONE |  | No Access |

# Appendix B – An Example Submission

<?xml version="1.0" encoding="UTF-8" ?>

- <esf:ESFSubmission xmlns:esf="http://www.for.gov.bc.ca/schema/esf" xmlns:gml="http://www.opengis.net/gml" xmlns:xlink="http://www.w3.org/1999/xlink" xmlns:mof="http://www.for.gov.bc.ca/schema/base" xmlns:abr="http://www.for.gov.bc.ca/schema/abr" xmlns:xsi="http://www.w3.org/2001/XMLSchema-instance" xsi:schemaLocation="http://www.for.gov.bc.ca/schema/esf http://www.for.gov.bc.ca/schema/esf/1/xsd/MOF/esf-submission.xsd http://www.for.gov.bc.ca/schema/abr http://www.for.gov.bc.ca/schema/fta/0/xsd/MOF/mof-abr.xsd">

- <esf:submissionMetadata>

<esf:emailAddress>mailto:John.Doe@MajorLicensee.com</esf:emailAddress>

<esf:telephoneNumber>6044445555</esf:telephoneNumber>

</esf:submissionMetadata>

- <esf:submissionContent>

- <abr:ABRSubmission>

<gml:description>A sample submission of As-Built Roads</gml:description>

<gml:name>Optional name.</gml:name>

- <gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

- <abr:submissionMetadataProperty>

- <abr:SubmissionMetadata>

<abr:validateOnly>true</abr:validateOnly>

<abr:contactName>Licensee contact Name</abr:contactName>

</abr:SubmissionMetadata>

</abr:submissionMetadataProperty>

- <abr:submissionItem>

- <abr:RoadTenure>

- <gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

<abr:forestFileId>A12345</abr:forestFileId>

<abr:timberMark>F16110</abr:timberMark>

<abr:cutBlockId>A</abr:cutBlockId>

- <abr:roadSectionMember>

- <abr:RoadSection>

- <gml:boundedBy>

<gml:null>unknown</gml:null>

</gml:boundedBy>

- <mof:spatialMetaData>

<mof:dataSource>datasource</mof:dataSource>

<mof:mostRecentSourceDate>2003-01-01</mof:mostRecentSourceDate>

<mof:captureMethod>capture method</mof:captureMethod>

<mof:observationDate>2003-01-01</mof:observationDate>

<mof:dataAccuracy>75</mof:dataAccuracy>

<mof:description>description</mof:description>

<mof:ccsmCode>GA9100032</mof:ccsmCode>

<mof:horizontalAccuracy>15</mof:horizontalAccuracy>

<mof:verticalAccuracy>10</mof:verticalAccuracy>

<mof:horizontalDatum>nad83</mof:horizontalDatum>

<mof:verticalDatum>cvd28</mof:verticalDatum>

<mof:coordinateSystemCode>1</mof:coordinateSystemCode>

</mof:spatialMetaData>

<abr:roadSectionId>1</abr:roadSectionId>

- <abr:centreLineOf>

- <gml:LineString srsName="EPSG:42102">

<gml:coordinates>0,0 100,0</gml:coordinates>

</gml:LineString>

</abr:centreLineOf>

- <abr:eventItem>

- <abr:bridge>

- <!-- Mandatory elements  -->

- <abr:location>

- <abr:distance>

<abr:from>10.3</abr:from>

<abr:to>10.4</abr:to>

</abr:distance>

</abr:location>

<abr:superStructureId>12345678</abr:superStructureId>

<abr:siteId>12345678</abr:siteId>

<abr:designLoadRating>100</abr:designLoadRating>

<abr:superStructureCode>type</abr:superStructureCode>

<abr:bridgeDeckCode>abc</abr:bridgeDeckCode>

<abr:substructureCode>abc</abr:substructureCode>

- <!--Optional elements  -->

<abr:fishPresent>true</abr:fishPresent>

<abr:riparianClass>true</abr:riparianClass>

<abr:fieldKmMarker>5</abr:fieldKmMarker>

<abr:streamName>stream name</abr:streamName>

<abr:currentLoadRating>100</abr:currentLoadRating>

<abr:designVehicleCode>abc</abr:designVehicleCode>

<abr:nextInspectionDate>2003-01-01</abr:nextInspectionDate>

<abr:bridgeDeckWidth>1</abr:bridgeDeckWidth>

<abr:chronicMaintenance>true</abr:chronicMaintenance>

<abr:upstreamRisk>true</abr:upstreamRisk>

<abr:inletRisk>true</abr:inletRisk>

<abr:adequateSize>true</abr:adequateSize>

<abr:historicWashout>true</abr:historicWashout>

</abr:bridge>

</abr:eventItem>

- <abr:eventItem>

- <abr:culvert>

- <!-- Mandatory elements  -->

- <abr:location>

- <abr:coordinate>

- <gml:Point srsName="EPSG:42012">

- <gml:coord>

<gml:X>30</gml:X>

<gml:Y>40</gml:Y>

</gml:coord>

</gml:Point>

</abr:coordinate>

</abr:location>

<abr:siteStructureId>1234567890</abr:siteStructureId>

<abr:diameterOrSpan>5000</abr:diameterOrSpan>

<abr:engineeredStructure>true</abr:engineeredStructure>

<abr:materialCode>abc</abr:materialCode>

<abr:designLoadRating>10</abr:designLoadRating>

- <!-- Optional elements  -->

<abr:fishPresent>true</abr:fishPresent>

<abr:riparianClass>abc</abr:riparianClass>

<abr:height>5000</abr:height>

<abr:streamName>stream name</abr:streamName>

<abr:currentLoadRating>10</abr:currentLoadRating>

<abr:designVehicleCode>abc</abr:designVehicleCode>

<abr:length>10</abr:length>

<abr:description>description</abr:description>

<abr:adequateSize>true</abr:adequateSize>

<abr:inletRisk>true</abr:inletRisk>

<abr:chronicMaintenance>true</abr:chronicMaintenance>

<abr:comments>comments</abr:comments>

<abr:siteId>1234567890</abr:siteId>

<abr:historicWashout>true</abr:historicWashout>

</abr:culvert>

</abr:eventItem>

- <abr:eventItem>

- <abr:linearStructure>

- <!-- Mandatory elements  -->

- <abr:location>

- <abr:coordinates>

- <abr:from>

- <gml:Point srsName="EPSG:42012">

- <gml:coord>

<gml:X>30</gml:X>

<gml:Y>40</gml:Y>

</gml:coord>

</gml:Point>

</abr:from>

- <abr:to>

- <gml:Point srsName="EPSG:42012">

- <gml:coord>

<gml:X>30</gml:X>

<gml:Y>50</gml:Y>

</gml:coord>

</gml:Point>

</abr:to>

</abr:coordinates>

</abr:location>

<abr:linearFeatureTypeCode>abc</abr:linearFeatureTypeCode>

<abr:engineeredStructure>true</abr:engineeredStructure>

<abr:linearStructureClassCode>FD</abr:linearStructureClassCode>

</abr:linearStructure>

</abr:eventItem>

- <abr:eventItem>

- <abr:organicMaterial>

- <!-- Mandatory elements  -->

- <abr:location>

- <abr:distance>

<abr:from>12</abr:from>

<abr:to>13</abr:to>

</abr:distance>

</abr:location>

<abr:roadFeatureTypeCode>abc</abr:roadFeatureTypeCode>

</abr:organicMaterial>

</abr:eventItem>

- <abr:eventItem>

- <abr:constructionDate>

- <!-- Mandatory elements  -->

- <abr:location>

- <abr:distance>

<abr:from>15</abr:from>

<abr:to>16</abr:to>

</abr:distance>

</abr:location>

<abr:constructionTypeCode>abc</abr:constructionTypeCode>

<abr:actualCompletionDate>2003-01-01</abr:actualCompletionDate>

- <!-- Optional elements  -->

<abr:plannedActivity>true</abr:plannedActivity>

<abr:seasonCode>abc</abr:seasonCode>

<abr:outstandingObligation>true</abr:outstandingObligation>

<abr:inspectionPlanned>true</abr:inspectionPlanned>

<abr:comments>comments</abr:comments>

</abr:constructionDate>

</abr:eventItem>

- <abr:eventItem>

- <abr:accessControl>

- <!-- Mandatory elements  -->

- <abr:location>

<abr:distance>20</abr:distance>

</abr:location>

<abr:accessControlCode>abc</abr:accessControlCode>

- <!-- Optional elements  -->

<abr:closureId>1234567890123456</abr:closureId>

<abr:installDate>2003-01-01</abr:installDate>

<abr:rationaleCode>obj code</abr:rationaleCode>

<abr:roadAccessMethodCode>veh use</abr:roadAccessMethodCode>

</abr:accessControl>

</abr:eventItem>

- <abr:eventItem>

- <abr:deactivation>

- <!-- Mandatory elements  -->

- <abr:location>

- <abr:distance>

<abr:from>30</abr:from>

<abr:to>40</abr:to>

</abr:distance>

</abr:location>

<abr:roadDeactivationLevelStatus>abc</abr:roadDeactivationLevelStatus>

<abr:actualCompletionDate>2003-01-01</abr:actualCompletionDate>

- <!-- Optional elements  -->

<abr:signageDate>2003-01-01</abr:signageDate>

<abr:revegDate>2003-01-01</abr:revegDate>

<abr:comments>comments</abr:comments>

</abr:deactivation>

</abr:eventItem>

</abr:RoadSection>

</abr:roadSectionMember>

</abr:RoadTenure>

</abr:submissionItem>

</abr:ABRSubmission>

</esf:submissionContent>

</esf:ESFSubmission>

# Appendix C – Troubleshooting

<This section to be completed>

A listing of system messages and appropriate actions for the user.