Electronic Submission Framework

FME Feature Representation – ABR Submission

| AUTHOR | DATE | COMMENTS |
| --- | --- | --- |
| Juan Chu Chow | 14 Jul 2004 | Original |

Feature Type: ESFSubmissionMetadata	4

Feature Type: ABRSubmissionMetadata	4

Feature Type: RoadTenure	4

Feature Type: RoadSection	5

Feature Type: Bridge	6

Feature Type: Culvert	8

Feature Type: LinearStructure	9

Feature Type: OrganicMaterial	10

Feature Type: ConstructionDate	12

Feature Type: AccessControl	13

Feature Type: Deactivation	14

Feature Type: SpatialMetaData	15

### Feature Type: ESFSubmissionMetadata

The Feature is required.

Comments:

Attributes:

| Name | # Type | Optional/Required | Description |
| --- | --- | --- | --- |
| emailAddress | char(100) | Required |  |
| telephoneNumber | decimal(10,0) | Required |  |

FME ESF Specific Attributes:

| Name | Type/Value | # Optional/Required | Description |
| --- | --- | --- | --- |
| gml2_type | gml2_no_geom | Required |  |

### Feature Type: ABRSubmissionMetadata

The Feature is required.

Comments:

Attributes:

| Name | # Type | Optional/Required | Description |
| --- | --- | --- | --- |
| validateOnly | boolean | Required |  |
| contactName | char(50) | Required |  |

Note:  We might as well attach the optional ‘description’ and ‘name’ attributes into this ABRSubmissionMetadata feature.

It does not make much sense to have an extra required feature just to hold these attributes which may never be used.

The ‘description’ and ‘name’ attributes are predefined optional GML attributes.

description		char(256)		Optional

name			char(64)		Optional

FME ESF Specific Attributes:

| Name | # Type/Value | Optional/Required | Description |
| --- | --- | --- | --- |
| gml2_type | gml2_no_geom | Required |  |

### Feature Type: RoadTenure

The Feature is required.  Submissions must contain at least one RoadTenure.

Comments:

A RoadTenure FME feature must be uniquely identified by its esf_road_tenure_id attribute since a submission may contain 1 or more RoadTenures.

A RoadTenure may have 1 or more RoadSections.  A RoadSection FME feature will also have an esf_road_tenure_id attribute identifying its containing RoadTenure.

Attributes:

| Name | Type | Optional/Required | Description |
| --- | --- | --- | --- |
| forestFileId | char(10) | Required |  |
| timberMark | char(6) | Optional |  |
| cutBlockId | char(10) | Required |  |

FME ESF Specific Attributes:

| # Name | # Type/Value | # Optional/Required | # Description |
| --- | --- | --- | --- |
| gml2_type | gml2_no_geom | Required |  |
| esf_road_tenure_id | some unique id | Required | Uniquely identifies the RoadTenure within the submission. |

### Feature Type: RoadSection

The Feature is required  A RoadTenure must have at least one RoadSection.

Comments:

Rereferences its RoadTenure container through its required esf_road_tenure_id attribute.

A RoadSection FME feature additionally contains an esf_road_section_id attribute which must be unique within the submission.

Attributes:

| Name | Type | Optional/Required | Description |
| --- | --- | --- | --- |
| roadSectionId | char(10) | Required |  |

FME ESF Specific Attributes:

| Name | Type/Value | Optional/Required | Description |
| --- | --- | --- | --- |
| gml2_type | gml2_line | Required |  |
| esf_road_section_id | some unique id | Required | Uniquely identifies the RoadSection within the submission. |
| esf_road_tenure_id | some unique id | Required | Must match a RoadTenure’s esf_road_tenure_id attribute in the submission. |
| esf_spatial_metadata_id | some id | Optional | Reference to the spatial metadata. |

### Feature Type: Bridge

The Feature is an optional Event.

Comments:

The Bridge FME feature may be either a line or a non-geometrical feature.  If the Bridge is a non-geometrical feature then its location.from and location.to attributes will be non-empty.

If the Bridge has line geometry, then its first coordinate will correspond to the start point of the event, and the second (and last) coordinate corresponds to the end point of the event.

Attributes:

| Name | Type | Optional/Required | Description |
| --- | --- | --- | --- |
| location.from | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| location.to | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| superStructureId | char(8) | Required |  |
| siteId | char(8) | Required |  |
| designLoadRating | decimal(10,0) | Required |  |
| superStructureCode | char(10) | Required | ?? Schema says: “superStructureCode”, Guide says: “superStructureType”. ??? |
| bridgeDeckCode | char(20) | Required | ??? name differs – bridgeDeckCode (schema), deckType (guide), also order differs in guide this property position is swapped w/ substructureCode ??? |
| substructureCode | char(14) | Required | ??? substructureCode (schema), substructureType (guide) |
| fishPresent | boolean | Optional |  |
| riparianClass | char(10) | Optional |  |
| fieldKmMarker | char(15) | Optional |  |
| streamName | char(30) | Optional |  |
| currentLoadRating | decimal(10,0) | Optional |  |
| designVehicleCode | char(10) | Optional |  |
| nextInspectionDate | date | Optional |  |
| bridgeDeckWidth | decimal(7,2) | Optional |  |
| chronicMaintenance | boolean | Optional |  |
| upstreamRisk | boolean | Optional |  |
| inletRisk | boolean | Optional |  |
| adequateSize | boolean | Optional |  |
| historicWashout | boolean | Optional |  |

FME ESF Specific Attributes:

| Name | Type/Value | Optional/Required | Description |
| --- | --- | --- | --- |
| gml2_type | gml2_line gml2_no_geom | Required | gml2_type is gml2_no_geom when the location of the event is provide through the location.from and location.to distance attributes. |
| esf_road_section_id | some unique id | Required | Reference its containing RoadSection. Its value must match the RoadSection’s esf_road_section_id attribute. |

### Feature Type: Culvert

The Feature is an optional Event.

Comments:

The Culvert FME feature may be either a point or a non-geometrical feature.  If the Culvert is a non-geometrical feature then its location.distance attribute will be non-empty.

Attributes:

| Name | Type | Optional/Required | Description |
| --- | --- | --- | --- |
| location.distance | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| siteStructureId | char(10) | Required |  |
| diameterOrSpan | decimal(9,3) | Required |  |
| engineeredStructure | boolean | Required | ??? Missing in the guide ??? |
| materialCode | char(10) | Required |  |
| designLoadRating | decimal(10,0) | Required |  |
| fishPresent | boolean | Optional |  |
| riparianClass | char(10) | Optional |  |
| height | decimal(7,2) | Optional |  |
| streamName | char(30) | Optional |  |
| currentLoadRating | decimal(10,0) | Optional |  |
| designVehicleCode | char(10) | Optional |  |
| length | decimal(7,2) | Optional |  |
| description | char(80) | Optional |  |
| adequateSize | boolean | Optional |  |
| inletRisk | boolean | Optional |  |
| chronicMaintenance | boolean | Optional |  |
| comments | char(4000) | Optional |  |
| siteId | char(10) | Optional |  |
| historicWashout | boolean | Optional |  |

FME ESF Specific Attributes:

| Name | Type/Value | Optional/Required | Description |
| --- | --- | --- | --- |
| gml2_type | gml2_point gml2_no_geom | Required | gml2_type is gml2_no_geom when the location of the event is provide through the location.distance attribute. |
| esf_road_section_id | some unique id | Required | Reference its containing RoadSection. Its value must match the RoadSection’s esf_road_section_id attribute. |

### Feature Type: LinearStructure

The Feature is an optional Event.

Comments:

The LinearStructure FME feature may be either a line or a non-geometrical feature.  If the LinearStructure is a non-geometrical feature then its location.from and location.to attributes will be non-empty.

If the LinearStructure has line geometry, then its first coordinate will correspond to the start point of the event, and the second (and last) coordinate corresponds to the end point of the event.

Attributes:

| Name | Type | Optional/Required | Description |
| --- | --- | --- | --- |
| location.from | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| location.to | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| linearFeatureTypeCode | char(20) | Required | ??? linearFeatureTypeCode (schema), type (guide) ??? |
| engineeredStructure | boolean | Required |  |
| linearStructureClassCode | char(20) | Required | ??? linearStructureClassCode (schema), code (guide) ??? |

FME ESF Specific Attributes:

| Name | Type/Value | Optional/Required | Description |
| --- | --- | --- | --- |
| gml2_type | gml2_line gml2_no_geom | Required | gml2_type is gml2_no_geom when the location of the event is provide through the location.from and location.to distance attributes. |
| esf_road_section_id | some unique id | Required | Reference its containing RoadSection. Its value must match the RoadSection’s esf_road_section_id attribute. |

### Feature Type: OrganicMaterial

The Feature is an optional Event.

Comments:

The OrganicMaterial FME feature may be either a line or a non-geometrical feature.  If the OrganicMaterial is a non-geometrical feature then its location.from and location.to attributes will be non-empty.

If the OrganicMaterial has line geometry, then its first coordinate will correspond to the start point of the event, and the second (and last) coordinate corresponds to the end point of the event.

Attributes:

| Name | Type | Optional/Required | Description |
| --- | --- | --- | --- |
| location.from | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| location.to | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| roadFeatureTypeCode | char(20) | Required | ??? roadFeatureTypeCode (schema), featureType (guide) ??? |

FME ESF Specific Attributes:

| Name | Type/Value | Optional/Required | Description |
| --- | --- | --- | --- |
| gml2_type | gml2_line gml2_no_geom | Required | gml2_type is gml2_no_geom when the location of the event is provide through the location.from and location.to distance attributes. |
| esf_road_section_id | some unique id | Required | Reference its containing RoadSection. Its value must match the RoadSection’s esf_road_section_id attribute. |

### Feature Type: ConstructionDate

The Feature is an optional Event.

Comments:

The ConstructionDate FME feature may be either a line or a non-geometrical feature.  If the ConstructionDate is a non-geometrical feature then its location.from and location.to attributes will be non-empty.

If the ConstructionDate has line geometry, then its first coordinate will correspond to the start point of the event, and the second (and last) coordinate corresponds to the end point of the event.

Attributes:

| Name | Type | Optional/Required | Description |
| --- | --- | --- | --- |
| location.from | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| location.to | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| constructionTypeCode | char(10) | Required | ??? constructionTypeCode (schema), activityCode (guide) ??? |
| actualCompletionDate | date | Required |  |
| plannedActivity | boolean | Optional |  |
| seasonCode | char(10) | Optional |  |
| outstandingObligation | boolean | Optional |  |
| inspectionPlanned | boolean | Optional |  |
| comments | char(4000) | Optional |  |

FME ESF Specific Attributes:

| Name | Type/Value | Optional/Required | Description |
| --- | --- | --- | --- |
| gml2_type | gml2_line gml2_no_geom | Required | gml2_type is gml2_no_geom when the location of the event is provide through the location.from and location.to distance attributes. |
| esf_road_section_id | some unique id | Required | Reference its containing RoadSection. Its value must match the RoadSection’s esf_road_section_id attribute. |

### Feature Type: AccessControl

The Feature is an optional Event.

Comments:

The AccessControl FME feature may be either a point or a non-geometrical feature.  If the AccessControl is a non-geometrical feature then its location.distance attribute will be non-empty.

Attributes:

| Name | Type | Optional/Required | Description |
| --- | --- | --- | --- |
| location.distance | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| accessControlCode | char(20) | Required | ??? accessControlCode (schema), accessControlFeature (guide) ??? |
| closureId | char(20) | Optional |  |
| installDate | date | Optional |  |
| rationaleCode | char(10) | Optional | ??? rationaleCode (schema), objectiveCode (guide) ??? |
| roadAccessMethodCode | char(10) | Optional | ??? roadAccessMethod (schema), vehicleUse (guide) ??? |

FME ESF Specific Attributes:

| Name | Type/Value | Optional/Required | Description |
| --- | --- | --- | --- |
| gml2_type | gml2_point gml2_no_geom | Required | gml2_type is gml2_no_geom when the location of the event is provide through the location.distance attribute. |
| esf_road_section_id | some unique id | Required | Reference its containing RoadSection. Its value must match the RoadSection’s esf_road_section_id attribute. |

### Feature Type: Deactivation

The Feature is an optional Event.

Comments:

The Deactivation FME feature may be either a line or a non-geometrical feature.  If the Deactivation is a non-geometrical feature then its location.from and location.to attributes will be non-empty.

If the Deactivation has line geometry, then its first coordinate will correspond to the start point of the event, and the second (and last) coordinate corresponds to the end point of the event.

Attributes:

| Name | Type | Optional/Required | Description |
| --- | --- | --- | --- |
| location.from | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| location.to | decimal(8,2) | Optional | Represents the location of the event as a distance from the road’s PoC. It is not present if the Event has geometry. |
| roadDeactivationLevelStatus | char(30) | Required | ??? roadDeactivationLevelStatus (schema), deactivationlevel (guide) ??? |
| actualCompletionDate | date | Required |  |
| signageDate | date | Optional |  |
| revegDate | date | Optional |  |
| comments | char(4000) | Optional |  |

FME ESF Specific Attributes:

| Name | Type/Value | Optional/Required | Description |
| --- | --- | --- | --- |
| gml2_type | gml2_line gml2_no_geom | Required | gml2_type is gml2_no_geom when the location of the event is provide through the location.from and location.to distance attributes. |
| esf_road_section_id | some unique id | Required | Reference its containing RoadSection. Its value must match the RoadSection’s esf_road_section_id attribute. |

### Feature Type: SpatialMetaData

The Feature is required.  Each Submission must have at least 1 RoadSection, and each RoadSection must have a related SpatialMetaData.

Comments:

The FME SpatialMetaData feature contains an esf_spatial_metadata attribute which uniquely identifies it within the submission.

??? The ABR spatial metadata differs from FTA and RESULTS, yet they seem to refer to the same version (version 1) of the MOF base schema. ???

Attributes:

| Name | Type | Optional/Required | Description |
| --- | --- | --- | --- |
| dataSource | char(10) | Optional |  |
| mostRecentSourceDate | date | Optional |  |
| captureMethod | char(10) | Optional |  |
| observationDate | date | Optional |  |
| dataAccuracy | decimal(2,0) | Optional | ??? XML Schema says NonNegativeInteger2D ??? |
| description | char(2000) | Optional |  |
| ccsmCode | char(10) | Optional |  |
| horizontalAccuracy | decimal(3,0) | Optional |  |
| verticalAccuracy | decimal(3,0) | Optional |  |
| horizontalDatum | char(10) | Optional |  |
| verticalDatum | char(10) | Optional |  |
| coordinateSystemCode | char(10) | Optional | ?? Present in XML Schema, mof-base.xsd, absent in the ABR SubmissionGuide??? |

FME ESF Specific Attributes:

| Name | Type/Value | Optional/Required | Description |
| --- | --- | --- | --- |
| gml2_type | gml2_no_geom | Required |  |
| esf_spatial_metadata_id | some id | Required | The id must be unique across the SpatialMetaDatas in a submission |